Pyrolysis

Goal: To demonstrate and understand the physics of fire – solid fuel is gasified with the application of heat and the gas itself burns

Fire intensity: Similar to a small fire without a flue

Space Requirements

- Outdoor recommended, unless fume hood available

  - Excellent outside demo outside unless wind is strong

  - If inside, must be under a fume hood

- Solid table

- Participants 5ft back from table

Supplies

- Camping stove & fuel

- Ring stand with clamp

- Distillation flask/flask with small outlet

- Wood shavings & funnel

- Lighter

- Tar catchment beaker

Personal Protective Equipment

- Protective goggles

- Nomex lab coat or jacket

- Leather gloves

- Long cotton/nomex pants

- Hard hat

- Fire extinguisher

Standard Operating Procedure

- Make sure distillation flask is empty of charred fuel

- Check distillation flask for any cracks/chips

- Make sure there is sufficient fuel for the demo (fuel canister not empty)

- Fill flask ½ way with wood shavings and cap with stopper

- Put on all PPE described above

- Set up flask on ring stand with stove underneath

  - Be sure there is no risk of flask tipping

    - Set up on flat, even surface

    - Make sure flask is not pulling ring stand over in a direction that is less stable

- Place drip catcher (small flask) directly under distillation tube (where pyrolyte will be exiting)

- Set up stove underneath flask

- Light stove

- Proceed with demo, making sure nobody gets too close

  - GLASS IS VERY HOT!

- When done, turn off stove, but leave it set up until it cools off naturally

- Empty out used fuel, and store back into padded protective box